This AHA is provided as an example of content to consider during development of project-specific AHAs for like activities

Activity: Flooring installation (carpet, sheet vinyl, VCT, tile)

| HAZARDS |  |  |
| --- | --- | --- |
| JOB STEP | HAZARDS | ACTIONS TO ELIMINATE OR MINIMIZE EACH HAZARD |
| Floor prep and layout: | Risk of exposure to asbestos from VCT/ mastic under lament | Reference results of ACM survey. |
| Install flooring | Risk of exposure to silica based products | Respirators or dust masks may be required. Respirator use in accordance with Respiratory Protection Program, including medical evaluation and fit testing. |
|  | Risk of fire from heat gun or open flamed tool | Follow the Hot Work Permit as required. Fire watch is required (trained in fire extinguishers). Employee shall have proper training on heat guns or open flamed tools. |
|  | Risk of injuries/ cuts from improper tool use or storage. | Employees are to be trained on the use of tools. Protect knifes or open blade with a holder or sheath to protect the blade while not in use. Keep all used razor blades in a sharps container labeled as "SHARPS” |